A user manual of the “GISnetwork3D”- An R package for 3D spatial network analysis

Table of Contents

1	Introduction	2

2	Code Demonstration	3

2.1	Data import	3

2.1.1	Import boundary of the study area	3

2.1.2	Import building data	3

2.1.3	Import 3D pedestrian network data	4

2.1.4	Import the location of primary schools	4

2.1.5	Import digital terrain model (DTM)	4

2.1.6	Plot the geography of the study area	4

2.2	Data preprocessing	5

2.2.1	Convert resBuild to centroid points and define the Z value	5

2.2.2	Define the z value of primary school	5

2.2.3	Topology correction of the pedestrian network data	5

2.2.3.1	Component identification	6

2.2.3.2	Snapping disconnected segments	6

2.2.3.3	Create the pedestrian network by extracting the edges and vertices	8

2.2.3.4	Identify the nearest network vertices for residential buildings and primary schools	9

2.2.3.5	Create network graph	9

2.3	Analysis	9

2.3.1	Analysis: Origin-Destination matrix and summary	9

2.3.1.1	Create OD matrix	10

2.3.1.2	Summarize OD matrix	10

2.3.2	Analysis: Deriving shortest path for each O-D pair	11

2.3.2.1	Create paths	11

2.3.2.2	Summary of paths	12

2.3.3	Analysis: Deriving service area	12

2.3.3.1	Create sample building	12

2.3.3.2	Create service area	12

2.3.3.3	Plot the service area	13

2.4	Function documentation	14

2.4.1	A brief summary of the functionalities	14

2.4.2	build3DNET - Build network for 3D line vector	14

2.4.3	calculate_slope - Calculate the degree slope	15

2.4.4	dist3D - Calculate the straight-line 3D distance between two points	15

2.4.5	findVertices3D - Find the nearest network vertex for each input point	16

2.4.6	GISigraph3D - Create igraph from edges and vertices produced from the function "build3DNET"	16

2.4.7	Identify_components - Identify the components of the network	16

2.4.8	ODmatrix3D - Create OD matrix	17

2.4.9	ODmatrix3D_summary - Summarizing OD matrix	17

2.4.10	reverseLine_by_condition - Reverse line by condition	18

2.4.11	ServiceArea3D - Create 3D service area	18

2.4.12	Shortest_3Dpaths_for_pairs - Create one or more paths for each pair of origin and destination	19

2.4.13	Shortest_3Dpath_oneToMany - Create paths from one origin to one or more destination(s)	19

2.4.14	snapNET3D - Connect disconnected line within a distance threshod	20

2.4.15	tobr_hf_pace - Calculate the walking speed according to the Tobler's hiking function	20

# Introduction

The "GISnetwork3D" is an R package supporting 3D spatial network analysis capable of reading, pre-processing, and analyzing 3D spatial data. This user manual provides a step-by-step demonstration of assessing the spatial accessibility to primary school in a small community in Hong Kong, China by using this package.

This demonstration selected a small community in the Eastern District on the Hong Kong Island as a study area and used the "GISnetwork3D" package to model the spatial accessibility to primary school from residential buildings.

We will use 3D building data, a Digital Terrain Model (DTM), the location of the primary school, 3D pedestrian network, and they were preprocessed in advance. Please note that the data is for demonstration only and is not intended for formal investigation!

The demonstration folder "workspace > Rexample" contained all of the documents and data needed to reproduce this demonstration. The details of the documents in this folders are listed below.

Rexample.Rproj: An R project. Open this R project and insert the R codes demonstrated below to reproduce the results.

Rexample.R: An R script can be imported into the R project by clicking "file > Open file… > Rexample.R". This R script recorded all of the codes for this demonstration.

data: The folder contains all the data for this demonstration

Export: The folder is a destination for depositing maps and plots created during the demonstration.

# Code Demonstration

This demonstration requires several R packages as follows. Please ensure they are installed locally and loaded.

library(sf)

library(raster)

library(GISnetwork3D)

library(furrr)

library(purrr)

library(igraph)

library(tmap)

library(scales)

Then, the "GISnetwork3D" package can be installed manually from the R project. Please open the R project called " GISnetwork3D.Rproj" which is located in workspace> rPackage > GISnetwork3D.

Then, on the top bar, please click "Build" and then "Install Package". The package will then be installed automatically.

## Data import

Objective: Import all necessary data for the analysis.

### Import boundary of the study area

We have two boundaries. First, the studyArea is the boundary of the study area of which the buildings (or the demands) are located.

Second, the studyAreaBuf is the boundary of the 2000m buffer of the study area. This buffer area accounts for the cross-boundary movement of the demand near the fringe of the study area. All data, except for the buildings, were delineated by this boundary. The raw data were retrieved from the District Council Constituency Areas (DCCA) census map, downloaded from an open data portal by the Hong Kong government (https://www.csdi.gov.hk/).

studyArea = st_read("data/studyArea.gpkg")

studyAreaBuf = st_read("data/studyAreaBuf.gpkg")

### Import building data

We have two building data. First is the resBuild. This includes residential buildings. Second, it is otherBuild. This includes buildings other than residential buildings, such as podium structures.

The raw data were retrieved from the iB1000 dataset, which was downloaded from an open data portal managed by the Hong Kong government (https://www.csdi.gov.hk/). The landuse of buildings is defined by the Land Utilization map 2022, downloaded from the CSDI portal (https://portal.csdi.gov.hk/geoportal/#metadataInfoPanel).

The building data are in polygon format. Though this data contains the third dimension (z value), the z value is 0 for all polygons. The true z value is recorded in the attribute table with the field name called "BASELEVEL".

resBuild = st_read("data/resBuild.gpkg")

otherBuild = st_read("data/otherBuild.gpkg")

### Import 3D pedestrian network data

This 3D pedestrian data recorded pedestrian paths in the study area in 3D. This dataset does not simply drape the network on the earth's terrain and record its surface height. This dataset records the actual location of each path. Therefore, this dataset also includes underground, footbridge, and paths inside some major shopping malls. The raw data were downloaded from an open data portal managed by the Hong Kong government (https://www.csdi.gov.hk/).

ped = st_read("./data/ped.gpkg")

### Import the location of primary schools

The location of the primary school within the buffer of the study area was retrieved from the iGeoCom dataset from the open data portal managed by the Hong Kong government (https://www.csdi.gov.hk/). This data records the 2D coordinates of the points of interest. Therefore, we will need to estimate the z value from the digital terrain model (DTM).

PRiScH = st_read("./data/PRiScH.gpkg")

### Import digital terrain model (DTM)

This data recorded the terrain of Hong Kong with 5m resolution in raster format. This raw dataset was obtained from the Hong Kong government (https://www.csdi.gov.hk/).

DTM = raster("./data/DTM.tif")

### Plot the geography of the study area

Now, we can create a map to better understand the geography of the study area. The created map will be saved in the "Export > map".

png("./Export/map/Map01_Geography_of_study_area.png", width = 180, height = 100, units = "mm", res = 600)

tm_shape(DTM) + tm_raster(title = "DTM(m)") +

tm_shape(studyAreaBuf %>% st_cast("MULTILINESTRING")) + tm_lines(col = "studyAreaBuf", lwd = 2, labels = "2,000m buffer of study area", title.col = "") +

tm_shape(ped) + tm_lines(col = "ped", title.col = "", labels = "Pedestrian path", palette = "grey", lwd = 0.6 ) +

tm_shape(studyArea %>% st_cast("MULTILINESTRING")) + tm_lines(col = "studyArea", lwd = 1.5, labels = "Study area", title.col = "", palette = "grey40", alpha = 0.5) +

tm_shape(otherBuild) + tm_fill(col = "otherBuild", labels = "Other buildings", title = "", palette = "grey70") +

tm_shape(resBuild) + tm_fill(col = "resBuild", labels = "Residential buildings", title = "", palette = "blue") +

tm_shape(PRiScH) + tm_dots(col = "PRiScH", size = 0.1, title = "", labels = "Primary school", palette = "purple", shape = 17) +

tm_compass(position = c("RIGHT", "TOP")) + tm_scale_bar() +

tm_layout(legend.outside = T)

dev.off()

## Data preprocessing

Objective: Preprocess data for latter spatial accessibility analysis

### Convert resBuild to centroid points and define the Z value

In this study, the spatial accessibility measures the access to primary school from resBuild. We will convert the resBuild to centroid point for each polygon and then use this centroid as the origins for the spatial accessibility analysis. We will create a 3D centroid point layer.

# Convert resBuild to point and drop the z value

resBuildPT = resBuild %>% st_zm() %>% st_centroid()

# Get the coordinate of the resBuildPT and convert it to data frame

resBuildPT = resBuildPT %>% cbind( st_coordinates(resBuildPT) ) %>% st_drop_geometry()

# Create the 3D centroid point. The BASELEVEL defines the z value.

resBuildPT$Z = resBuildPT$BASELEVEL

resBuildPT = resBuildPT %>% st_as_sf(coords = c("X", "Y", "Z"), crs = 2326)

### Define the z value of primary school

As the primary school data lack the Z value. Therefore, we extracted the Z value from the DTM and converted the PRiScH to 3D point features.

# Extract Z value

PRiScH$Z = raster::extract(DTM, PRiScH)

# Get the X Y coordinate of PRiScH and drop geometry

PRiScH = PRiScH %>% cbind( st_coordinates(PRiScH) ) %>% st_drop_geometry()

# Create 3D point layer

PRiScH = PRiScH %>% st_as_sf(coords = c("X", "Y", "Z"), crs = 2326)

### Topology correction of the pedestrian network data

This section will show how to use the "GISnetwork3D" to correct some geometry errors of the input 3D line layer.

First, we will demonstrate the component identification. Second, we will demonstrate how to connect the disconnected network nodes.

#### Component identification

In a network, each component represents a group of connected network nodes. Each component is isolated, meaning components A and B are not connected.

#We will use the "Identify_components" function from the "GISnetwork3D" package to identify components for the ped layer. The input is the 3D line feature and the output is the input plus an additional field of "component". The component is a numeric vector. Each unique value represents the identifier of each isolated group of line segments.

ped = ped %>% Identify_components()

# Here, we used table function to count the frequency of each component and the total number of component

# frequency of each component

table(ped$component)

# total number of component. We can see that there are 47 components in the pedestrian path.

table(ped$component) %>% length()

We can map the component to inspect what happened. We can define a group as the main component (mainComp). mainComp = 1 if the line is within the main component and 0 otherwise.

ped$mainComp = 1

ped$mainComp[ped$component > 1] = 0

tm_shape(ped) + tm_lines(col = "mainComp", style = "cat", palette = c("red", "black"))

#From the map, we can see that some components are isolated, which can be related to the clipping of the network by the buffer of the study area and also because of digitization errors, such as overshoot and undershoot.

#### Snapping disconnected segments

Inaccurate routing results may emerge if the lines supposed to connect are disconnected. We can define a distance threshold. Then a 3D line is created to connect two disconnected vertices within this threshold.

We can use the "snapNET3D" function from the "GISnetwork3D" package to do this task.

We can use an example to better demonstrate the functionality

We created two lines (LineA and LineB). Both lines have the same X and Y coordinates but the Z coordinates. The difference in Z value for the first points is 1 meter while the second points is 0.1 m. We created a small line to connect the second points of the two lines.

LineA = data.frame( X = c(838150, 838140), Y = c(815228.3, 815227.4), Z = c(0, 0), L1 = 1) # Vertices of Line A

LineB = data.frame( X = c(838150, 838140), Y = c(815228.3, 815227.4), Z = c(1, 0.1), L1 = 2) # Vertices of Line B

lines = LineA %>% rbind(LineB) %>% st_as_sf(coords = c("X", "Y", "Z"), crs = 2326) # Combine vertices of Lines A and B

lines = qgis_run_algorithm("native:pointstopath", INPUT = lines, GROUP_EXPRESSION = "L1", OUTPUT_TEXT_DIR =  tempdir())$OUTPUT %>% st_read() # Create lines from vertices

# We can use the "snapNET3D" function to conenct vertices within a threshold 3D distance. The example belows connected vertices within 0.1m

snapLines = lines %>% snapNET3D(0.1)

print(snapLines) # We can inspect the snapLines and see two additional lines with the field of "snap" as 1. This indicates that the line is a snapping line.

snapLines %>% st_coordinates() # We can then inspect the coordinates of the lines. We can see that the two snapping lines are identical but in opposite directions. This is to avoid routing errors when we choose directional routing.

Then, we snapped vertices within 0.5m for the pedestrian network

pedSnapped = ped %>% dplyr::select(-component, -mainComp) %>% snapNET3D(threshold = 0.5) # snap vertices within 0.5m

pedSnapped$snap %>% sum # We created 2266 snapping lines, equivalent to connecting 1095 pairs of vertices

pedSnapped = pedSnapped %>% Identify_components() # Identify components

length(table(ped$component)) - length(table(pedSnapped$component)) # Now we can compare the snapped and unsnapped ped and the snapping reduced 8 components

pedSnapped$mainComp = 1

pedSnapped$mainComp[pedSnapped$component > 1] = 0

# We can create a map to compare the distribution of disconnected components between snapped and unsnapped ped

png("./Export/map/Map02_disconnected_snapped_vs_unsnapped_comparison.png", width = 180, height = 100, units = "mm", res = 600)

tm_shape(ped %>% subset(mainComp == 0)) + tm_lines(col = "mainComp", style = "cat", palette = "red", lwd = 1.5, title.col = "", labels = "Disconnected(unsnapped)") +

tm_shape(pedSnapped %>% subset(mainComp == 0)) + tm_lines(col = "mainComp", style = "cat", palette = "green", title.col = "", labels = "Disconnected(snapped)") +

tm_compass(position = c("RIGHT", "TOP")) + tm_scale_bar() +

tm_layout(legend.outside = T)

dev.off()

# >>>>> For smooth routing, we will only retain the main component for later routing

pedSnapped = pedSnapped %>% subset(component == 1)

#### Create the pedestrian network by extracting the edges and vertices

In network analysis, we need to have the information of the edges and nodes/vertices. Each edge connects two nodes. Edges record the weight/cost. The edges and vertices layers are the prerequisites for constructing the graph for routing and network analysis.

Here, we create two sets of networks. One is a usual network, and the other is an anisotropic network.

We can use the "build3DNET" function from the "GISnetwork3D" package for this task.

The "build3DNET" will produce a list with edges and vertice layers. By default, the argument "ped" is FALSE. This means the network built has the direction of each edge based on the digitization direction. In later routing, you can still use directional or bi-directional routing. If "ped" is TRUE, a pedestrian network is created. This acknowledges the usual bidirectional nature of the pedestrian route and the direction-dependent weight, known as anisotropic movement. By enabling this argument, the output network will calculate the walking duration for each line segment, which depends on direction. Each line segment has two travel times depending on the direction. The calculation is based on Tobler's hiking function. Besides, If a column "directCond" is presented in the attribute table of the input line, directed edges will be filtered before producing a bi-directional network when the ped argument is TRUE. Only 1 and 0 are accepted for the directCond. 1 indicates that the edge is directed according to its digitization direction. 0 indicates that it is bi-directed, and bi-directed walking speed and travel time were calculated for these edges. This can account for some portion of pedestrian paths are not bi-direction while others are bi-direction with directional-dependent weight. We did not considered both directional and bi-directional pedestrian network in this demonstration. Further information can be found in the package documentation.

# Create the usual network

pedNET = pedSnapped %>% build3DNET()

# Create an anisotropic network

pedNETa = pedSnapped %>% build3DNET(ped = TRUE)

We can print the edges layer of the output of pedNET and pedNETa. We can see that the first two columns are "from" and "to". They correspond to the direction of each line and the connected vertices. The number corresponds to the vID of the vertices layers of the output of pedNET and pedNETa. We can see that the pedNETa$edges has two additional fields: the walking pace and the "second", which refers to the duration of walking through the edge. Other additional information produced by this function includes the unique ID of each edge (eID), the slope in degree, and the length of each edge in 3D and 2D.

pedNET$edges

pedNETa$edges

pedNET$vertices

pedNETa$vertices

#### Identify the nearest network vertices for residential buildings and primary schools

The "findVertices3D" function from the "GISnetwork3D" package helps find the nearest network vertices for each input 3D point layer. The input is two 3D point layers; one is the point of interest, and the other is the vertices layer from the "build3DNET" function.

# Find nearest network vertices for resBuildPT

resBuildPT = resBuildPT %>% findVertices3D(pedNET$vertices)

# Find nearest network vertices for PRiScH

PRiScH = PRiScH %>% findVertices3D(pedNET$vertices)

# We can print the output, and we can see that there is an additional field "vID". This is the unique ID of the network vertex nearest to the input POI.

print(resBuildPT)

print(PRiScH)

#### Create network graph

The final step is to create an igraph for later routing using the "GISigraph3D" function from the "GISnetwork3D" package.

The input is the network edges and vertices created previously.

# Create an undirected graph

pedGraph_iso = GISigraph3D(pedNET$edges, pedNET$vertices, directed = FALSE)

# Create an anisotropic graph

pedGraph_aniso = GISigraph3D(pedNETa$edges, pedNETa$vertices, directed = TRUE)

## Analysis

### Analysis: Origin-Destination matrix and summary

This section shows how to use the "GISnetwork3D" to calculate the least-cost travel between each origin and each destination. Origin-Destination Matrix (ODM) is a data matrix recording the cost of travel between each origin and each destination. Each row represents each origin, and each column represents each destination.

Before starting, we must give each POI a unique ID in ascending chronological order.

resBuild$ID = 1:nrow(resBuild)

resBuildPT$ID = 1:nrow(resBuild)

PRiScH$ID = 1:nrow(PRiScH)

#### Create OD matrix

We will calculate the OD matrix for each residential building and each primary school for both isotropic and anisotropic routing using the "ODmatrix3D" function from the "GISnetwork3D" package.

For better comparison, we need to calculate the travel time for the bi-directional pedestrian network. As it does not consider the direction, the slope will become useless. Therefore, we assume 0 degrees of slope and use the 3D length to calculate the travel time.

# According to Tobler's hiking function, the walking pace at 0-degree slope is 0.71m/s

pedNET$edges$second = pedNET$edges$eLength3D * 0.71

Here, we created three scenarios. First, the travel time between each O-D based on an undirected movement and assumed a 0-degree slope. Second is the travel time from each residential building to each school. Third is the travel time from each school to each residential building. This directional movement is defined by the argument "mode" as "in" or "out" - by default, it is "all", assuming bi-directional.

# Create ODM for undirected isotropic movement

ODMiso = ODmatrix3D(pedGraph_iso, oID = resBuildPT$ID, oV = resBuildPT$vID, dID = PRiScH$ID, dV = PRiScH$vID, weight = pedNET$edges$second)

# Create ODM for anisotropic movement

ODManiso_from =  ODmatrix3D(pedGraph_aniso, oID = resBuildPT$ID, oV = resBuildPT$vID, dID = PRiScH$ID, dV = PRiScH$vID, weight = pedNETa$edges$second, mode = "out") # ODM according to leaving from the building tower to the school

ODManiso_to =  ODmatrix3D(pedGraph_aniso, oID = resBuildPT$ID, oV = resBuildPT$vID, dID = PRiScH$ID, dV = PRiScH$vID, weight = pedNETa$edges$second, mode = "in") # ODM according to moving to the building tower from the school

#### Summarize OD matrix

We need to summarize the OD matrix for each origin (here, it is the residential building). We can use the "ODmatrix3D_summary" function from the "GISnetwork3D" package to do this task.

# This function can calculate the mean, median travel cost, and availability within a cost threshold. In the example below, we counted the number of schools within 600s walks with a weight of availability for each destination as 1.

ODMiso_summary = ODmatrix3D_summary(ODMiso, oID = resBuildPT$ID, oV = resBuildPT$vID, dID = PRiScH$ID, dV = PRiScH$vID, threshold = 600, avaiW = rep(1, nrow(PRiScH)))

ODManiso_from_summary = ODmatrix3D_summary(ODManiso_from, oID = resBuildPT$ID, oV = resBuildPT$vID, dID = PRiScH$ID, dV = PRiScH$vID, threshold = 600, avaiW = rep(1, nrow(PRiScH)))

ODManiso_to_summary = ODmatrix3D_summary(ODManiso_to, oID = resBuildPT$ID, oV = resBuildPT$vID, dID = PRiScH$ID, dV = PRiScH$vID, threshold = 600, avaiW = rep(1, nrow(PRiScH)))

Here, we ploted three scatter plots to compare travel time for the three scenarios mentioned above.

We can see the variation in travel time between scenarios. Besides, we can also see that isotropic routing tends to have lower values than anisotropic routing. We can also see the variability in travel time between travel from and to residential buildings.

png("./Export/plot/Plot01_ODM_time_comparison.png", width = 180, height = 60, units = "mm", res = 600)

par(mfrow = c(1, 3), mar = c(4.1, 4.1, 0.5, 0.5))

plot(ODManiso_from_summary$minCost, ODManiso_to_summary$minCost, pch = 21, col = alpha("grey", 0.5), bg = alpha("red", 0.5), xlab = "Travel time (from)", ylab = "Travel time (to)", xlim = c(120, 800), ylim = c(0, 730))

plot(ODMiso_summary$minCost, ODManiso_to_summary$minCost, pch = 21, col = alpha("grey", 0.5), bg = alpha("red", 0.5), xlab = "Travel time (isotropic)", ylab = "Travel time (to)", xlim = c(120, 800), ylim = c(0, 730))

plot(ODMiso_summary$minCost, ODManiso_from_summary$minCost, pch = 21, col = alpha("grey", 0.5), bg = alpha("red", 0.5), xlab = "Travel time (isotropic)", ylab = "Travel time (from)", xlim = c(120, 800), ylim = c(0, 730))

dev.off()

### Analysis: Deriving shortest path for each O-D pair

In the previous section, we summarized the ODM. The ODM summary recorded each origin's destination with the least-cost travel time. We can use this O-D pair and derive the path in the 3D line feature using the "Shortest_3Dpaths_for_pairs" function from the "GISnetwork3D" package.

#### Create paths

# Path of isotropic walking

paths_iso = Shortest_3Dpaths_for_pairs(pedGraph_iso, edges = pedNET$edges, oID = ODMiso_summary$oID, oV = ODMiso_summary$oV, dID = ODMiso_summary$dID, dV = ODMiso_summary$dV, weight = pedNET$edges$second)

# Path of walking from the origin

paths_from = Shortest_3Dpaths_for_pairs(pedGraph_aniso, edges = pedNETa$edges, oID = ODManiso_from_summary$oID, oV = ODManiso_from_summary$oV, dID = ODManiso_from_summary$dID, dV = ODManiso_from_summary$dV, weight = pedNETa$edges$second, mode = "out")

# Path of walking toward the origin

paths_to = Shortest_3Dpaths_for_pairs(pedGraph_aniso, edges = pedNETa$edges, oID = ODManiso_to_summary$oID, oV = ODManiso_to_summary$oV, dID = ODManiso_to_summary$dID, dV = ODManiso_to_summary$dV, weight = pedNETa$edges$second, mode = "in")

# We can print one of the examples. We can see that the exported data are the edges of the line layers with an additional field oID. The oID is the ID of the origin.

print(paths_to)

#### Summary of paths

We can summarize the exported path. For instance, we can calculate the mean walking pace or summarize environmental variables along the way. Here, we calculate the average walk pace.

# We use paths_to as example.

paths_toSummary = paths_to

paths_toSummary = aggregate(walkpace ~ oID, data = paths_toSummary %>% st_drop_geometry(), FUN = mean ) # average the walk pace

# We can see that some routes having faster pace while have slower.

summary(paths_toSummary$walkpace)

Min. 1st Qu.  Median    Mean 3rd Qu.    Max.

0.8075  0.8445  0.8702  1.1374  1.1583  4.4840

The "Shortest_3Dpath_oneToMany" function from the "GISnetowrkGIS" is similar to the "Shortest_3Dpaths_for_pairs" while the former supports one-to-one and one-to-many routing.

### Analysis: Deriving service area

This section demonstrates how to use the "ServiceArea3D" function from the "GISnetwork3D" package to explore the reachable areas by an origin point.

We will create a service area for a sampled building using both isotropic and anisotropic methods.

#### Create sample building

sampleBuild = resBuild[1, ]

sampleBuildPT = resBuildPT[1, ]

#### Create service area

# We created 10-minute catchments

# Path of isotropic walking

catchment_iso = ServiceArea3D(pedGraph_iso, edges = pedNET$edges, vertices = pedNET$vertices, oID = sampleBuildPT$ID, oV = sampleBuildPT$vID, weight = pedNET$edges$second, search = 2000, costThreshold = 600)

# Catchment of walking from the origin

catchment_from = ServiceArea3D(pedGraph_aniso, edges = pedNETa$edges, vertices = pedNETa$vertices, oID = sampleBuildPT$ID, oV = sampleBuildPT$vID, weight = pedNETa$edges$second, search = 2000, costThreshold = 600, mode = "out")

# Catchment of walking toward the origin

catchment_to = ServiceArea3D(pedGraph_aniso, edges = pedNETa$edges, vertices = pedNETa$vertices, oID = sampleBuildPT$ID, oV = sampleBuildPT$vID, weight = pedNETa$edges$second, search = 2000, costThreshold = 600, mode = "in")

class(catchment_to) # The output is a list.

The output list consists of a line layer showing the reachable edges and a data frame listing all reachable vertices. This function first calculates the cost between the origin and all network vertices, and then subsets all vertices within reachable cost, and then extracts the edges containing these vertices.

#### Plot the service area

png("./Export/map/Map03_serviceArea_sample.png", width = 180, height = 100, units = "mm", res = 600)

tm_shape(catchment_iso$edgesReached) + tm_lines() +

tm_shape(DTM) + tm_raster(title = "DTM(m)", breaks = c(0, 20, 40, 80, 100, 150, 200, 1000) ) +

tm_shape(ped) + tm_lines(col = "ped", title.col = "", labels = "Pedestrian path", palette = "grey", lwd = 0.6 ) +

tm_shape(catchment_iso$edgesReached) + tm_lines(col = "oID", title.col = "", labels = "Bi-directional", palette = "grey20") +

tm_shape(catchment_from$edgesReached) + tm_lines(col = "oID", title.col = "", labels = "Outward", palette = "red", lwd = 2) +

tm_shape(catchment_to$edgesReached) + tm_lines(col = "oID", title.col = "", labels = "Toward", palette = "green", lwd = 0.85) +

tm_shape(sampleBuild) + tm_fill(col = "res", title = "", labels = "Sample building", palette = "purple") +

tm_compass(position = c("LEFT", "TOP")) + tm_scale_bar() +

tm_layout(legend.outside = T)

dev.off()

From the result, we can see that the elevation increases southward. The walking pace is faster for going downhill than going uphill. Therefore, the outward catchment extends northward while the inward catchment retreats southward. The isotropic travel (assuming 0 slopes) gets the greatest areal footprint.

## Function documentation

### A brief summary of the functionalities

| Functionality | Function | Brief details |
| --- | --- | --- |
| Geometry & topology correction/ modification | snapNET3D | - Connect disconnected line vertices situated within a predefined 3D distance - Addressing overshoot/ undershoot problems |
|  | reverseLine_by_condition | - Reverse all or indexed line segments - Correct the conflict of direction between true and digitizing direction |
| Network creation | Build3DNET | - Decompose a 3D line layer into two layers: edges and vertices - Support the creation of anisotropic (bi-directional routing with direction-dependent weight)/ partial anisotropic (some paths are directed while some are bi-directional with anisotropic weights) network |
|  | findVertices3D | - Identify the nearest network vertices for the input 3D point - A prerequisite of routing as it identifies the entry point to the network for each POI |
|  | GISgraph3D | - Convert the edges and vertices layers from the "Build3DNET" function to an "igraph" object for routing |
| Analysis | ODmatrix3D | - Calculate the travel cost between each origin and each destination - Support anisotropic/ partial anisotropic routing |
|  | ODmatrix3D_summary | - Summary statistics on the OD matrix for each origin - Support the calculating the weighted availability |
|  | Shortest_3Dpaths_for_pairs | - Extract the path in a 3D line layer for each origin-destination pair - Support anisotropic/ partial anisotropic routing |
|  | Shortest_3Dpath_oneToMany | - Extract the path in a 3D line layer from one origin to one or more destinations - Support anisotropic/ partial anisotropic routing |
|  | ServiceArea3D | - Identify edges and vertices within reach for a 3D point - Support anisotropic/ partial anisotropic routing |
| Utility | calculate_slope | - Calculate the degree slope between two points in 3D |
|  | dist3D | - Calculate the distance between two points in 3D |
|  | tobr_hf_pace | - Calculate the walking speed according to Tobler's hiking function based on the input degree slope |
|  | Identify_components | - Classify line segments into components. - Each component is an isolated set of lines |

### build3DNET - Build network for 3D line vector

# Description

Build network for 3D line vector.

# Usage

build3DNET(line, ped = F)

# Arguments

line

A sf line feature with the z value.

ped

Boolean: Indicate whether the network is built as a pedestrian network. If TRUE, this function will create a network that supports bi-directional routing with slope/directional dependent walking time(s) for each edge.

# Details

This function produces the edges and vertices of the input line feature.

Each edge is only connected by two vertices as this function first explodes the line feature. The edges record the from and to vertices, of which the index is the vID recorded in the vertices layer. The digitization direction determines the from and to vertices. A unique ID is generated for each edge segment (eID) and each vertex (vID).

If the ped is TRUE, the produced edges will record the slopes and the walking time for both directions. The walking time is based on Tobler's hiking function.

If a column "directCond" is presented in the attribute table of the input line, directed edges will be filtered before producing a bi-directional network when the ped argument is TRUE. Only 1 and 0 are accepted for the directCond. 1 indicates that the edge is directed according to its digitization direction. 0 indicates that it is bi-directed, and bi-directed walking speed and travel time were calculated for these edges.

### calculate_slope - Calculate the degree slope

# Description

Calculate the degree slope.

# Usage

calculate_slope(x1, y1, z1, x2, y2, z2)

# Arguments

x1

A numeric vector of X coordinate of the from point.

y1

A numeric vector of Y coordinate of the from point.

z1

A numeric vector of Z coordinate of the from point.

x2

A numeric vector of X coordinate of the to point.

y2

A numeric vector of Y coordinate of the to point.

z2

A numeric vector of Z coordinate of the to point.

### dist3D - Calculate the straight-line 3D distance between two points

# Description

Calculate the straight-line 3D distance between two points.

# Usage

dist3D(x1, y1, z1, x2, y2, z2)

# Arguments

x1

A numeric vector of X coordinate of the from point.

y1

A numeric vector of Y coordinate of the from point.

z1

A numeric vector of Z coordinate of the from point.

x2

A numeric vector of X coordinate of the to point.

y2

A numeric vector of Y coordinate of the to point.

z2

A numeric vector of Z coordinate of the to point.

### findVertices3D - Find the nearest network vertex for each input point

# Description

Find the nearest network vertex for each input point.

# Usage

findVertices3D(pt, vertices)

# Arguments

pt

A sf point feature with Z value.

vertices

The vertices of a network produced from the function - "build3DNET"

# Details

This function finds the nearest vertex from the vertices layer, produced from the function "build3DNET", for each input "pt". The input "pt" must have the z value. The output has the original pt with the vID of the nearest network vertex.

### GISigraph3D - Create igraph from edges and vertices produced from the function "build3DNET"

# Description

Create igraph from edges and vertices produced from the function "build3DNET".

# Usage

GISigraph3D(edges, vertices, directed = FALSE)

# Arguments

edges

Edges produced from the function "build3DNET"

vertices

Vertices produced from the function "build3DNET"

directed

Boolean: If TRUE, a directed graph is produced.

# Details

This function convert the "edges" and "vertices" to igraph object for network analysis. The input "edges" and vertices should be the output of "build3DNET". Or a line layer of "edges" and a point layer of "vertices" with the Z dimension. The "edges" must record the vertex ID of the from and to vertices in the first two columns. The "vertices" should have a field "vID" in the first column, which should be a vector of chronological order (1,2,3...n).

### Identify_components - Identify the components of the network

# Description

Identify the components of the network.

# Usage

Identify_components(line)

# Arguments

line

A sf line feature with the z value.

# Details

This function identifies the component to which each edge belongs for the input line layer. This function first converts the input line layer into network data using the "build3DNET" function, and it is then converted to igraph using the "GISigraph" function to identify the component of edges. Please be cautious that the output layer is an exploded version of the line, meaning each line segment only has two connected vertices.

### ODmatrix3D - Create OD matrix

# Description

Create OD matrix.

# Usage

ODmatrix3D(graph, oID, oV, dID, dV, weight, mode = "all")

# Arguments

graph

An igraph object created from the function "GISigraph3D"

oID

A numeric vector of the ID of the origin location(s)

oV

A numeric vector of the vertex ID of the origin location(s)

dID

A numeric vector of the ID of the destination location(s)

dV

A numeric vector of the vertex ID of the destination location(s)

weight

A numeric vector of weight/ cost for each edge for the least-cost routing.

mode

The direction of routing: "all" is undirected, "in" is toward the origin, and "out" is from the origin.

# Details

This function produce an OD matrix that calculate the cost of the least-cost paths between each origin and destination.

### ODmatrix3D_summary - Summarizing OD matrix

# Description

Summarizing OD matrix.

# Usage

ODmatrix3D_summary(frame, oID, oV, dID, dV, threshold = 500, avaiW)

Arguments

frame

A dataframe of OD matrix.

oID

A numeric vector of the IDs of the origin locations

oV

A numeric vector of the vertices IDs of the origin locations

dID

A numeric vector of the IDs of the destination locations

dV

A numeric vector of the vertices IDs of the destination locations

threshold

A number indicating the cost threshold for defining the catchment for calculating the availability of destinations.

avaiW

A numeric vector defining the weight for each destinatioin for calculating the availability within a threshold travel cost from each origin location. Weight of one for a destination refers to counting a destination as one if it is within the catchment of the origin within a cost budget.

# Details

This function summarizes the OD matrix produced from the function "ODmatrix3D". This function calculates the minimum cost, mean cost, median cost, and availability for each origin to many destinations. The availability is calculated by the sum of "avaiW" of each destination for each origin within a "threshold" search. avaiW is the weight for each destination. For instance, one means that the destination is counted as 1. The threshold is the cost <= which the destination is counted as available within a certain level of cost.

### reverseLine_by_condition - Reverse line by condition

# Description

This function reverses the direction of each line for the sf line features.

# Usage

reverseLine_by_condition(line, condition = -999)

# Arguments

line

A sf line feature.

condition

A numeric vector (1 or 0) defining which line requires reversion. The input length must be the same as the total number of rows of lines. Value of 1 = requiring reversion. 0 = not requiring reversion. All line segments are reversed if the condition is not specified by default.

# Details

This function reverses the direction of each line for the sf line features. The output line layer has an additional column called reversed. 0 means no reversion, and 1 means reversed.

### ServiceArea3D - Create 3D service area

# Description

Create 3D service area.

# Usage

ServiceArea3D(graph, edges, vertices, oID, oV, mode = "all", weight, search = 500, costThreshold = 500)

# Arguments

graph

An igraph object created from the function "GISigraph3D"

edges

A sf line layer with the fields of "from" and "to" to indicate the IDs of vertices for the start and end of each line segment. This object is preferably the "edges" output from the function "build3DNET"

vertices

A sf point layer with the fields of "vID".

oID

A numeric vector of the IDs of the origin locations

oV

A numeric vector of the vertices IDs of the origin locations

mode

The direction of routing: "all" is undirected, "in" is toward the origin, and "out" is from the origin

weight

A numeric vector of weight/ cost for each edge for the least-cost routing

search

A number indicating the size of straight-line radius used to first identify possible reachable vertices. This is expressed as 2D distance

costThreshold

The catchment size expressed as the maximum 3D network cost.

# Details

This function identifies vertices reachable within a cost threshold from an origin vertex and also all edges linking to these vertices. Therefore, the output is a list of (1) sf point showing the reachable vertices, and (2) the edges within threshold cost from each origin location.

### Shortest_3Dpaths_for_pairs - Create one or more paths for each pair of origin and destination

# Description

Create one or more paths for each pair of origin and destination.

# Usage

Shortest_3Dpaths_for_pairs(graph, edges, oID, oV, dID, dV, weight, mode = "all")

# Arguments

graph

An igraph object created from the function "GISigraph3D"

edges

A sf line layer with the fields of "from" and "to" to indicate the IDs of vertices for the start and end of each line segment. This object is preferably the "edges" output from the function "build3DNET"

oID

A numeric vector of the IDs of the origin locations

oV

A numeric vector of the vertices IDs of the origin locations

dID

A numeric vector of the IDs of the destination locations

dV

A numeric vector of the vertices IDs of the destination locations

weight

A numeric vector of weight/ cost for each edge for the least-cost routing

mode

The direction of routing: "all" is undirected, "in" is toward the origin, and "out" is from the origin

# Details

This function extracts the edges of shortest path for each OD pair.

### Shortest_3Dpath_oneToMany - Create paths from one origin to one or more destination(s)

# Description

Create paths from one origin to one or more destination(s).

# Usage

Shortest_3Dpath_oneToMany(graph, edges, weight, oV, dV, mode = "all")

# Arguments

graph

An igraph object created from the function "GISigraph3D"

edges

A sf line layer with the fields of "from" and "to" to indicate the IDs of vertices for the start and end of each line segment. This object is preferably the "edges" output from the function "build3DNET"

weight

A numeric vector of weight/ cost for each edge for the least-cost routing

oV

A numeric vector of the vertex ID of the origin location

dV

A numeric vector of the vertex/vertices ID(s) of the destination location(s)

mode

The direction of routing: "all" is undirected, "in" is toward the origin, and "out" is from the origin

# Details

This function create separate shortest path for each O and D pair in a one-to-many basis.

### snapNET3D - Connect disconnected line within a distance threshod

# Description

Connect disconnected line within a distance threshod.

# Usage

snapNET3D(line, threshold = 0.5)

# Arguments

line

A sf line feature with the z value.

threshold

A distance threshold within which the disconnected vertices are to be linked by a small edge.

# Details

This function connects the disconnected lines that are separated from each other within a predefined distance in 3D. Please note that the output layer is an exploded version of the line. Each line segment only has two connected vertices.

### tobr_hf_pace - Calculate the walking speed according to the Tobler's hiking function

# Description

Calculate the walking speed according to the Tobler's hiking function.

# Usage

tobr_hf_pace(Deg)

# Arguments

Deg

A numeric vector in degree slope.

# Examples

## The function is currently defined as

function (Deg)

{

S = tan(Deg * pi/180)

result = 0.6 * exp(3.5 * abs(S + 0.05))

return(result)

}